Lab-09

Design a fuzzy logic-based expert system.

Implement fuzzification, rule evaluation, and defuzzification.

Objectives:

The purpose of this lab is to:

- Understand the concepts of fuzzification and defuzzification.

- Learn how fuzzy systems convert real-world values into fuzzy sets (fuzzification).

- Explore how rules are applied to fuzzy values (rule evaluation).

- Learn how fuzzy results are turned back into clear, crisp outputs (defuzzification).

- Implement a simple fuzzy system for temperature control (like an Air Conditioner).

Theory:
When computers or machines take decisions, they usually work with crisp values (clear numbers).

- Example: If temperature > 30 → Turn AC ON, else OFF.
But in real life, decisions are not always black and white.

At 28°C, should the AC be ON or OFF? Maybe it should run a little bit. This “in-between” thinking is where fuzzy logic helps.

### Small Analogy:

Think about your mom making tea:

- If tea is too sweet → add more water.

- If tea is not sweet enough → add sugar.

- But sometimes it’s "just a little too sweet" → she adds just a little water.
That’s fuzzy logic in real life!

Fuzzification:

Fuzzification means changing exact input values into fuzzy categories using membership functions.

Example: Temperature

- Cold: 0–15°C

- Warm: 15–30°C

- Hot: 30–45°C

If the temperature = 25°C →

- Cold = 0

- Warm = 0.8

- Hot = 0.2

So, 25°C is mostly “Warm” but a little “Hot.”

Example 1:

Example 2: Assigning fuzzy values to temperature

### Rule Evaluation:

We use IF–THEN rules to make decisions.

Rules:

- IF Cold → Fan Speed = Low

- IF Warm → Fan Speed = Medium

- IF Hot → Fan Speed = High

At 25°C:

- Warm = 0.8 → Medium Fan = 60%

- Hot = 0.2 → High Fan = 90%

Example 3: fuzzy fan control system

### Defuzzification:

Defuzzification means converting fuzzy outputs into one exact (crisp) value that a machine can actually use.

It works like taking an average based on importance. For example:

- Medium fan (60%) has weight 0.8

- High fan (90%) has weight 0.2

So,
Fan Speed = (0.8 × 60) + (0.2 × 90) = 66%

This means at 25°C, the AC fan will run at 66% speed.

So, at 25°C, AC fan runs at 66% speed.

# Why Do We Need It?

- Fuzzy logic gives us answers in degrees (0.2, 0.7, etc.).

- Machines need one crisp value to work.

- Defuzzification is the translator between fuzzy logic and real actions.

Example: fuzzy fan control system (same problem with defuzzification)

### Common Methods of Defuzzification

### 1. Centroid (Balance Point)

### Think of all answers sitting on a seesaw. The point where it balances is the final decision.

### Example: hot=0.7, warm=0.3 → Fan ≈ 75%

### 2. Maximum Membership (Pick the Strongest)

### Pick the answer with the highest score.

### Example: hot=0.7, warm=0.3 → Fan = 90% (Hot)

### 3. Mean of Maximum (Average of Winners)

### If two answers tie, take the average.

### Example: warm=0.7, hot=0.7 → Fan = 75%

### 4. Smallest of Maximum

### If there is a tie, choose the smaller one.

### Example: warm=0.7, hot=0.7 → Fan = 60%

### 5. Largest of Maximum

### If there is a tie, choose the bigger one.

### Example: warm=0.7, hot=0.7 → Fan = 90%

### Key Points:

- Fuzzification = Convert numbers into fuzzy words.

- Rule Evaluation = Apply IF–THEN rules.

- Defuzzification = Convert fuzzy results back into numbers.

- Fuzzy logic makes machines think like humans in uncertain situations.

### Real-World Applications:

- AC & Refrigerator control systems (adjust cooling smoothly).

- Washing machines (decide wash time based on “little dirty” or “very dirty” clothes).

- Self-driving cars (decide actions based on “close,” “far,” or “very close” distances).

- Medical expert systems (doctor-like apps saying “mild fever,” “high fever”).

- Robotics (smooth movement instead of jerky ON/OFF actions).

- Recommendation systems (suggesting movies/music as “maybe like,” “probably like,” “definitely like”).

Lab Tasks:

Air Conditioner Control System

- Design a fuzzy system that adjusts AC cooling based on temperature and humidity.

- Define fuzzy sets for temperature (Cool, Comfortable, Hot) and humidity (Low, High).

- Define fuzzy sets for cooling power (Low, Medium, High).

- Write rules combining temperature and humidity.

- Show output cooling levels for three different input cases.